Continious Assignment 2 Report

Liu Xiangyu G1802061L

- Initialization

  - Calculate original RGA, RNGA, NI

Original RGA=

1.0986   -0.0876   -0.0110

0.0006    0.1045    0.8949

-0.0992    0.9831    0.1161

Original RNGA=

1.0339   -0.0100   -0.0240

0.0022    0.1035    0.8944

-0.0361    0.9065    0.1296

Original NI=

8.4716

  - Do pairing

According to original RGA, RNGA, NI, do pairing based on RNGA. Loop 1-1, 2-3 and 3-2 are chosen to be paird.

  - Calculate RGA RNGA NI RARTafter pairing

Paird RGA=

1.0986   -0.0876   -0.0110

-0.0992    0.9831    0.1161

0.0006    0.1045    0.8949

Paird RNGA=

1.0339   -0.0100   -0.0240

-0.0361    0.9065    0.1296

0.0022    0.1035    0.8944

Paird NI=

1.0199

RART =

0.9412    0.1139    2.1739

0.3640    0.9221    1.1163

3.5506    0.9905    0.9994

- BLT Decentralized Controller

  - Design Steps and Results

- Find the ultimate gain Ku and ultimate frequency u for each diagonal using relay experiment method.

Ultimate gain Ku =

-63.0317

-1.0610

2.9610

Ultimate Frequency Wu =

2.2125

0.1559

1.8845

- Calculate the expressions of Kci, tIi and tDi according to the formula.

- Calculate the expressions of G(jw) and Lc(jw).

- First Iteration Loop, to calculate the optimal F.

F =

1.8400

- Second Iteration Loop, give the optimal F, to calculate the optimal Fd.

Fd =

16.3900

- Export parameters to Simulink blocks.

kpBlt =

-15.5711   -0.2621    0.7315

kiBlt =

-3.5760   -0.0042    0.1431

kdBlt =

-0.2764   -0.0660    0.0152

- Execute simulation.

  - Figures

- ETF Decentralized Controller

  - Design Steps and Results

    - Find ETFs for diagonal loops according to Decentralized ETF rules.

    - Specify an Am and Calculate PID parameters by GPM Method.

kpDec =

-18.5675   -0.3919    1.0055

kiDec =

-0.2785   -0.0090    0.3516

kdDec =

0   -4.2603    0.7189

    - Export parameters to Simulink blocks.

  - Figures

- ETF Sparse Controller

  - Design Steps and Results

- Calculate the index matrix

X =

1.0000    0.0096    0.0232

0.0398    1.0000    0.1430

0.0024    0.1157    1.0000

- Specify a value of epsilon and threshold the index matrix to find which loops to control. Since all off-diagonal index are less than the empirical value 0.15, 0.10 is specified.

G0 =

1.0000         0         0

0    1.0000    0.1430

0    0.1157    1.0000

- Find ETFs for all loops whose G0 value is non-zero.

- Calculate PID parameters of ETF.

kpSprs =

-18.5675         0         0

0   -0.3919   -0.8703

0   -0.0332    1.0055

kiSprs =

-0.2785         0         0

0   -0.0090   -0.1828

0   -0.0026    0.3516

kdSprs =

0         0         0

0   -4.2603   -1.0357

0         0    0.7189

  - Figures

- ETF Decoupling Controller

  - Design Steps and Results

    - Using Normalized Decoupling Method, find GI  = inv(Gs)*GR, diagonal elements of Gs chosen as GR.

    - Specify time delay for gRs and gIs to maintain causality.

Calculate PID parameters of ETFs of GR with specified time delay.

kpDecp =

-2.5712

-0.5765

0.3004

kiDecp =

-0.0386

-0.0133

0.1050

kdDecp =

0

-6.2665

0.2148

    - Export PID parameters and GI transfor functions to Simulink blocks.

  - Figures

- Appendix I: Matlab Scripts

  - BLT.m

%relay experiment results

h=1;

a(1)=-0.0202;

a(3)=0.43;

a(2)=-1.2;

Pu(1)=18.2552-15.4154;

Pu(3)=16.6099-13.2757;

Pu(2)=101.5974-61.2889;

Wu=zeros(3,1);

Ku=zeros(3,1);

for i=1:3

Wu(i)=2*pi/Pu(i);

Ku(i)=4*h/(pi*a(i));

end

syms s;

Gjw=zeros(3,3);

Kzn=zeros(3,1);

tzn=zeros(3,1);

tdzn=zeros(3,1);

Kc=zeros(3,1);

ti=zeros(3,1);

td=zeros(3,1);

F=0;

Fd=0;

%calculate optimal F

BLT_1

%calculate optimal Fd

BLT_2

%get the pid parameters

for i=1:3

kpBlt(i)=Kc(i);

kiBlt(i)=Kc(i)/ti(i);

kdBlt(i)=Kc(i)*td(i);

end

  - BLT_1.m

for F=1:0.01:15

%calculate Kc ti

for i=1:3

Kzn(i)=Ku(i)/2.2;

tzn(i)=2*pi/(1.2*Wu(i));

Kc(i)=Kzn(i)/F;

ti(i)=F*tzn(i);

end

%calculate Gjw Gcjw and Lc

for k=1:100

w=pi/100*k;

s=w*j;

for m=1:3

for n=1:3

Gjw(m,n)=K(m,n)/(T(m,n)*s+1)^O(m,n)*exp(-L(m,n)*s);

end

end

for i=1:3

Gcjw(i,i)=Kc(i)*(1+1/(ti(i)*j*w));

end

Wjw=-1+det(I+Gjw*Gcjw);

Lc_=20*log10(abs(Wjw/(1+Wjw)));

if Lc_<=6

Lc(k)=20*log10(abs(Wjw/(1+Wjw)));

end

end

Lcm=[Lcm; F max(Lc)];

end

%find the optimal F

[v id]=max(Lcm(:,2));

F=Lcm(id,1)

  - BLT_2.m

for Fd=1:0.01:20

for i=1:3

Kzn(i)=Ku(i)/2.2;

tzn(i)=2*pi/(1.2*Wu(i));

tdzn(i)=0.125*Pu(i);

Kc(i)=Kzn(i)/F;

ti(i)=F*tzn(i);

td(i)=tdzn(i)/Fd;

end

for k=1:100

w=pi/100*k;

s=w*j;

for m=1:3

for n=1:3

Gjw(m,n)=K(m,n)/(T(m,n)*s+1)^O(m,n)*exp(-L(m,n)*s);

end

end

for i=1:3

Gcjw(i,i)=Kc(i)*(1+1/(ti(i)*j*w)+td(i)*j*w);

end

Wjw=-1+det(I+Gjw*Gcjw);

Lc_=20*log10(abs(Wjw/(1+Wjw)));

if Lc_<=6

Lc(k)=20*log10(abs(Wjw/(1+Wjw)));

end

end

Lcm=[Lcm; Fd max(Lc)];

end

[v id]=max(Lcm(:,2));

Fd=Lcm(id,1)

  - ETFDecentralized.m

AmDec=ones(3,1)*4;

for i=1:3

for j=1:3

if abs(RGA(i,j))<1

kij=K(i,j)/RGA(i,j);

else

kij=K(i,j);

end

if RART(i,j)>1

Zeta(i,j)=Zeta(i,j)/RART(i,j);

Tij=T(i,j)*RART(i,j);

Lij=L(i,j)*RART(i,j);

else

Tij=T(i,j);

Lij=L(i,j);

end

if i==j

if O(i,j)==1

kpDec(i)=pi*Tij/(2*AmDec(i)*Lij*kij);

kiDec(i)=pi/(2*AmDec(i)*Lij*kij);

kdDec(i)=0;

elseif O(i,j)==2

if kij==K(i,j)

kij=kij*sign(RGA(i,j));

end

kpDec(i)=pi*Zeta(i,j)/(AmDec(i)*wn(i,j)*Lij*kij);

kiDec(i)=pi/(2*AmDec(i)*Lij*kij);

kdDec(i)=pi/(2*AmDec(i)*wn(i,j)^2*Lij*kij);

end

end

end

end

  - ETFDecoupling.m

AmDecp=[3,3,3];

clear s;

s=tf('s');

phim=pi/4;

%get Gs and GR

Gs_=s;

GR=[s 0 0;0 s 0;0 0 s];

for i=1:3

for j=1:3

Gs_(i,j)=K(i,j)/(T(i,j)*s+1)^O(i,j);

end

GR(i,i)=K(i,i)/(T(i,i)*s+1)^O(i,i);

end

%calculate GI

GI=inv(Gs_)*GR;

%specify L

L_=L.*RART;

for i=1:3

LR(i,i)=max(L_(i,:));

end

for i=1:3

for j=1:3

GI(i,j)=GI(i,j)*exp(-(LR(i,i)-L_(i,j))*s);

end

GR(i,i)=GR(i,i)*exp(-LR(i,i)*s);

end

%calculate PID parameters

kpDecp(1)=pi*T(1,1)/(2*AmDecp(1)*LR(1,1)*K(1,1));

kiDecp(1)=pi/(2*AmDecp(1)*LR(1,1)*K(1,1));

kdDecp(1)=0;

for i=2:3

kpDecp(i)=T(i,i)*pi/(AmDecp(i)*LR(i,i)*K(i,i));

kiDecp(i)=pi/(2*AmDecp(i)*LR(i,i)*K(i,i));

kdDecp(i)=pi*T(i,i)^2/(2*AmDecp(i)*LR(i,i)*K(i,i));

end

  - ETFSparse.m

%caculate the index matrix

for i=1:3

for j=1:3

X(i,j)=abs(RNGA(i,j)/RNGA(i,i));

if X(i,j)>1

X(i,j)=1/X(i,j);

end

end

end

eps=0.1;

for i=1:3

for j=1:3

if X(i,j)<eps

G0(i,j)=0;

else

G0(i,j)=X(i,j);

end

end

end

AmSprs=ones(3)*4;

kpSprs=zeros(3,3);

kiSprs=zeros(3,3);

kdSprs=zeros(3,3);

%calculate PID parameters for each ETFs

for i=1:3

for j=1:3

if abs(RGA(i,j))<1

kij=K(i,j)/RGA(i,j);

else

kij=K(i,j);

end

if RART(i,j)>1

Zeta(i,j)=Zeta(i,j)/RART(i,j);

Tij=T(i,j)*RART(i,j);

Lij=L(i,j)*RART(i,j);

else

Tij=T(i,j);

Lij=L(i,j);

end

if G0(i,j)~=0

if O(i,j)==1

kpSprs(j,i)=pi*Tij/(2*AmSprs(i,j)*Lij*kij);

kiSprs(j,i)=pi/(2*AmSprs(i,j)*Lij*kij);

kdSprs(j,i)=0;

elseif O(i,j)==2

if kij==K(i,j)

kij=kij*sign(RGA(i,j));

end

kpSprs(j,i)=pi*Zeta(i,j)/(AmSprs(i,j)*wn(i,j)*Lij*kij);

kiSprs(j,i)=pi/(2*AmSprs(i,j)*Lij*kij);

kdSprs(j,i)=pi/(2*AmSprs(i,j)*wn(i,j)^2*Lij*kij);

end

end

end

end

- Appendix II: Simulink Models

  - Overall Structure

  - Open loops

  - BLT

  - ETF Decentralized Controller

  - ETF Sparse Controller

  - ETF Decoupling Controller

- Appendix III: Comparing Figures

  - Y1

  - Y2

  - Y3